RCBO

NB1L-63 Residual Current Operated Circuit Breaker

with over-current protection(Magnetic)

1. General

1.1 Certificates: CB, CE, FIMKO, SEMKO, PCT, UKREST;

1.2 Electric ratings:up to 63A, 230V/1P+N, 2P, 400V/3P, 3P+N, 4P, AC50/60Hz;

1.3 Tripping characteristics: B, C, D;

1.4 Icn: 1~40A: 6000A/10000A

50~63A: 6000A

1.5 Rated residual current operated making & breaking capacity

I  m: 500 A;

1.6 Rated residual non-operated current I  no: 0.5I  n;

1.7 Rated insulation voltage Ui : 500 V;

1.8 Rated impulse withstand voltage Uimp: 6000 V;

1.9 Life (operations):

a. Electrical life: 2000, COS   =0.85~0.9

b. Mechanical life: 2000

c. Operating frequency: 240/h(In   25A); 120/h(In   25A);

1.10 Standard: IEC/EN 61009-1.

2. Type Designation

N   B1L

Frame level rated current Derivative code (electro-magnetic residual current operated circuit breaker) Design sequence number

Miniature circuit breaker

Company code

3. Technical Data

3.1 Technical parameters

| NB1L-63, 1P+N |  |  |
| --- | --- | --- |
| 2 |  | N |

| N 1 T | In (A) | I n (mA) | CTN | Order Code |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Curve B | Curve C | Curve D |
|  | 1 | 30 | 48 | 142001 | 142166 | 142331 |
|  | 1 | 100 | 48 | 142002 | 142167 | 142332 |
|  | 1 | 300 | 48 | 142003 | 142168 | 142333 |
|  | 2 | 30 | 48 | 142004 | 142169 | 142334 |
|  | 2 | 100 | 48 | 142005 | 142170 | 142335 |
|  | 2 | 300 | 48 | 142006 | 142171 | 142336 |
|  | 3 | 30 | 48 | 142007 | 142172 | 142337 |
|  | 3 | 100 | 48 | 142008 | 142173 | 142338 |
|  | 3 | 300 | 48 | 142009 | 142174 | 142339 |
|  | 4 | 30 | 48 | 142010 | 142175 | 142340 |
|  | 4 | 100 | 48 | 142011 | 142176 | 142341 |
|  | 4 | 300 | 48 | 142012 | 142177 | 142342 |
|  | 6 | 30 | 48 | 142013 | 142178 | 142343 |
|  | 6 | 100 | 48 | 142014 | 142179 | 142344 |
|  | 6 | 300 | 48 | 142015 | 142180 | 142345 |

.31.	.32.

.38.